CyFj11: FlowJo v11 Workspace Import and Legacy Format Export for R-Based Flow Cytometry Analysis

# Title Page

Title: CyFj11: FlowJo v11 Workspace Import and Legacy Format Export for R-Based Flow Cytometry Analysis

Short running title: CyFj11: FlowJo v11 / R Interoperability

Manuscript version: v0.2-draft (2026-05-22)

Authors: Bernd Jagla, bernd.jagla@pasteur.fr (1)

Slobodan Culina, Slobodan.culina@pasteur.fr (2)

Francois Le Guerroue, francois.le-guerroue@pasteur.fr (2)

Esma Karkeni, esma.karkeni@pasteur.fr (2)

Milena Hasan, milena.hasan@pasteur.fr (2)

Affiliations: 1: Institut Pasteur, scBiomarkers UTechS & Bioinformatics & Biostatistics Hub, 25-28 rue du Docteur Roux, 75015 Paris

2 : Institut Pasteur, scBiomarkers UTechS, 25-28 rue du Docteur Roux, 75015 Paris

Corresponding author: Bernd Jagla, ORCID: https://orcid.org/0000-0002-7696-0484

Acknowledgments: [TODO: funding sources, acknowledgments]

Funding: [TODO: grant numbers and funding bodies]

Conflict of interest: The authors declare no conflicts of interest.

Data and code availability: The CyFj11 package is available at https://github.com/C3BI-pasteur-fr/CyFj11.

AI use disclosure: AI-assisted tools were used . All scientific content and interpretation are the responsibility of the authors.

Author contributions (CRediT): BJ: Conceptualization, Methodology, Software, Validation,

Investigation, Writing – original draft. MH: Supervision, Writing – review & editing

# Abstract

High-dimensional flow cytometry data analysis increasingly requires movement between the commercial FlowJo platform and the open-source R/Bioconductor ecosystem. The release of FlowJo version 11 (v11) introduced a JSON-based workspace format that is incompatible with existing R interoperability tools, creating a barrier for computational cytometry workflows. We present CyFj11, an R package that enables full import of FlowJo v11 workspaces into R GatingSet objects and export to the FlowJo v10 XML format. Import supports all major gate types (rectangle, polygon, ellipsoid, range, quadrant, and Boolean) and all common transformation functions (biexponential, logicle, log, arcsinh, and linear), as well as compensation matrix extraction. Export targets the FlowJo v10 XML format, which is Gating-ML 2.0 compliant and supported by current FlowJo versions, providing a practical interoperability pathway while avoiding the technical barriers of the undocumented v11 JSON schema. The asymmetric design of CyFj11 is intentional: v11 import is complete, while export uses the well-specified v10 standard. CyFj11 is implemented in pure R, requires no compiled code or container infrastructure, and runs on all platforms supported by R including macOS on Apple Silicon. The package is available at https://github.com/C3BI-pasteur-fr/CyFj11.

Keywords: flow cytometry, FlowJo, GatingSet, Gating-ML, workspace interoperability, R/Bioconductor

# Introduction

Flow cytometry is a cornerstone technology in cell biology, immunology, and clinical diagnostics, enabling simultaneous measurement of multiple markers across millions of individual cells (1). As panel sizes grow — from conventional 8–18 parameter fluorescence configurations to modern spectral cytometry panels exceeding 40 parameters — the analytical demands have correspondingly increased. Manual gating in commercial software remains the dominant practice for initial data exploration, while computational R-based pipelines are increasingly used for downstream statistical analysis, batch processing, and reproducible reporting.

FlowJo (BD Biosciences) is one of the most widely used commercial platforms for flow cytometry data analysis, providing a graphical interface for gating, compensation, and population statistics (2). The R/Bioconductor ecosystem, centered on the flowCore (3) and flowWorkspace (4) packages, provides a complementary suite of tools for programmatic analysis: automation of repetitive tasks, integration with statistical frameworks, and reproducible computational workflows (5). Effective interoperability between these two environments — enabling a researcher to import a FlowJo gating strategy into R, or to export R-derived gates back into FlowJo for visual review — is therefore essential for many laboratories.

The release of FlowJo version 11 introduced a fundamental change in workspace architecture. The XML-based .wsp format used in v10, which conforms to the ISAC Gating-ML 2.0 standard [4], was replaced by a JSON-based .flowjo format distributed as a ZIP archive. This new format uses extensive UUID cross-referencing among population definitions, sample-specific gate instances, data sources, and group assignments (see Supplementary File 1 for a detailed comparison). Because the v11 JSON schema is proprietary and not publicly documented, existing interoperability tools cannot read or write v11 workspaces.

The primary existing tool for FlowJo–R interoperability, CytoML, supports reading FlowJo v10 workspaces but not v11 (6). Write support in CytoML relies on compiled C++ components distributed via Docker containers, which have not been maintained for current operating system versions, including macOS on Apple Silicon (ARM architecture). These factors create a practical barrier for researchers who work with FlowJo v11 and need to integrate their analyses with R.

Hereby we present CyFj11, an R package that addresses this gap through asymmetric format conversion: full import support for FlowJo v11 workspaces into R GatingSet objects, and export to the FlowJo v10 XML format. The asymmetry is intentional — v11 import was implemented by reverse-engineering the ZIP-based JSON structure, while export targets the documented Gating-ML 2.0 v10 standard rather than the opaque v11 schema. FlowJo v11 can open v10-format workspaces, making this a practical workflow: import v11 for R analysis, export to v10 for return to FlowJo. The pure R implementation requires no compiled code or containers, ensuring cross-platform compatibility.

The tool is positioned as a complement to, rather than a replacement for, CytoML: it covers the v11 import case that CytoML does not handle, and provides a maintained native-R v10 export path.

Figure 1: CyFj11 architecture and data flow. The package enables bidirectional conversion between FlowJo v11 workspaces and R GatingSet objects. (A) Import workflow: FlowJo v11 .flowjo files are parsed via ZIP extraction and JSON parsing, then converted to GatingSet objects with compensation, transformations, and FCS data. (B) Export workflow: GatingSet objects are extracted to gates, populations, and samples, then converted to FlowJo v10 XML format. The exported .wsp file can be opened in FlowJo v11 (C).

# The CyFj11 Package

The CyFj11 package was developed to bridge the interoperability gap between FlowJo v11 and the R/Bioconductor ecosystem through a three-function workflow: read_flowjo11_workspace() imports FlowJo v11 workspaces, fj11_to_gatingset() converts them to GatingSet objects, and export_flowjo10_workspace() exports GatingSet objects to FlowJo v10 XML format.

## Design Rationale

CyFj11 provides asymmetric conversion: complete FlowJo v11 import, with export limited to the FlowJo v10 XML format (Figure 1). This design reflects a deliberate engineering decision, not an incomplete implementation. The FlowJo v11 JSON schema requires maintaining a consistent graph of UUID relationships across dozens of entity types; without vendor documentation, generating a valid v11 export would require full reverse-engineering of this schema with no guarantee of compatibility across FlowJo updates. In contrast, the v10 XML format follows the published Gating-ML 2.0 standard, enabling confident generation of valid, interoperable files. A detailed technical justification is provided in Supplementary File 1.

## Core Workflow

The typical CyFj11 workflow consists of three steps:

library(CyFj11)

# Step 1: Read a FlowJo v11 workspace

ws <- read_flowjo11_workspace("experiment.flowjo")

# Step 2: Convert to a GatingSet (resolves FCS files, applies compensations

#         and transformations, executes gates)

gs <- fj11_to_gatingset(ws, group_name = "All Samples")

# Step 3: Export back to FlowJo v10 format

export_flowjo10_workspace(gs, output_path = ".", workspace_name = "exported")

The read_flowjo11_workspace() function parses the ZIP archive and extracts the JSON workspace structure. The fj11_to_gatingset() function locates FCS files, applies the compensation matrix, maps transformations, and constructs a named list of per-sample GatingSet objects, preserving any sample-specific gate variations from FlowJo v11's tailored gate mechanism. Because certain properties — including per-sample gate adjustments and transformation parameters — can differ between samples, the result is a list rather than a single merged GatingSet. Users can combine samples for downstream analysis using flowWorkspace::merge_list_to_gs(), with the understanding that this step may discard per-sample variation. The export_flowjo10_workspace() function generates Gating-ML 2.0–compliant XML that FlowJo v11 can open. We have also included a function to extract the population counts and statistics stored in the FlowJo v10 .wsp files for convenience (extract-flowjo-stats).

## Supported Features

Table 1 summarizes the gate types and transformation functions supported by CyFj11 for import and export operations. All types supported on import can be inspected and reanalyzed in R; the export column indicates whether that type can be written back to a v10 workspace.

Table 1. Gate types and transformations supported by CyFj11.

| Feature | Import (v11 → R) | Export (R → v10) |
| --- | --- | --- |
| Gate types |  |  |
| Rectangle (1D and 2D) | Yes | Yes |
| Polygon | Yes | Yes |
| Ellipsoid | Yes | Yes |
| Range (1D) | Yes | No^a |
| Quadrant | Yes | No^b |
| Boolean (AND/OR/NOT) | Yes | Yes |
| Transformations |  |  |
| Biexponential (biexp) | Yes | Yes |
| Logicle | Yes | Yes |
| Log | Yes | Yes |
| Arcsinh (fasinh) | Yes | Yes |
| Linear | Yes | Yes |
| Other |  |  |
| Compensation matrix | Yes | Yes |
| Per-sample (tailored) gates | Yes | Merged to group^c |
| Sample keywords / metadata | Yes | Yes |

a Range gates are converted to 1D rectangle gates on export. b Quadrant gates have no direct v10 equivalent and are excluded from export with a warning. c Per-sample gate variations are not expressible in v10 format; the group-level gate is used.

## Additional Functionality

In addition to core conversion functionality, CyFj11 provides utility functions for workspace inspection and debugging. The pretty_print_flowjo() function extracts and formats the JSON content of a .flowjo file to a human-readable text file, which is useful for debugging and workspace inspection. Verbose logging can be enabled with set_verbose(TRUE) to trace conversion steps during troubleshooting; the current verbosity state can be queried with get_verbose().

## Installation

CyFj11 requires R ≥ 4.3 and Bioconductor packages flowCore and flowWorkspace. The package is available from GitHub:

# Install devtools if needed

install.packages("devtools")

# Install CyFj11 from GitHub

devtools::install_github("C3BI-pasteur-fr/CyFj11")

A Bioconductor release is planned following peer review.

# Results

We validated CyFj11 using 13 structured test scenarios designed to exercise all supported gate types and transformations in isolation and in combination (Table 2; full descriptions in Supplementary File 2). We selected these scenarios to cover all supported gate types (rectangle, polygon, ellipsoid, range, quadrant, boolean) in isolation and in combination with each transformation type (linear, biexponential, log, arcsinh). Scenarios were designed to exercise edge cases including hierarchical gating, Boolean combinations, and mixed transformation workflows.

For each scenario, synthetic FCS files were generated, a FlowJo v11 workspace was constructed programmatically, converted to a GatingSet via fj11_to_gatingset(), and exported to v10 format via export_flowjo10_workspace(). Population counts from R were taken as the ground truth for each scenario. FlowJo-side validation (opening the exported v10 workspace in FlowJo and comparing reported counts) is described below.

Test workspaces, FCS files, and analysis scripts are deposited at [10.5281/zenodo.20849817].

## Population Count Preservation — R-Side Baseline

Table 2 reports the population counts produced by R for each test scenario. These serve as the expected values for downstream FlowJo comparison. Of the 13 validation scenarios, 8 representative scenarios covering all major gate types are shown; complete results for all scenarios are provided in Supplementary Table S1.

Table 2. Population counts from R GatingSet across 13 validation scenarios.

| Test | Gate type | Transform | Population | R count |
| --- | --- | --- | --- | --- |
| 01 | None (root only) | Linear | root | 5,000 |
| 02 | 1D rectangle | Linear | FSC_filter | 4,776 |
| 03 | 2D rectangle | Linear | cells | 4,631 |
| 04 | Polygon | Linear | singlets | 7,555 |
| 05 | Ellipsoid | Linear | ellipse_cells | 7,301 |
| 06 | 1D rectangle | Biexponential | FITC_pos | 2,403 |
| 07 | 1D rectangle | Log | PE_pos | 3,198 |
| 08 | 1D rectangle | Arcsinh | APC_pos | 1,606 |
| 09 | Hierarchical (2 levels) | Linear | cells / singlets | 9,441 / 8,800 |
| 10 | Boolean AND | Linear | FSC_gate / SSC_gate / both | 7,554 / 7,392 / 6,979 |
| 11 | Mixed hierarchy (biexp + arcsinh + boolean) | Mixed | cells / FITC_pos / APC_pos / double_pos | 9,696 / 2,897 / 1,930 / 1,928 |
| 12 | Ellipsoid + range + AND/OR/NOT | Log + biexp | cells / FITC_hi / PE_hi / double_hi / either_hi / neither | 9,765 / 2,934 / 3,831 / 2,879 / 3,886 / 5,879 |
| 13 | 3-level hierarchy (polygon/rect/ellipse/boolean) | Arcsinh | live / singlets / FITC_pos / double_pos / not_double | 9,939 / 9,470 / 2,838 / 1,887 / 7,583 |

Note: Population names follow FlowJo convention where gate names reflect the marker or cell population being identified (e.g., "FSC_filter" for forward scatter filter, "FITC_pos" for FITC-positive cells).

## Population Count Preservation — FlowJo Comparison

[Text to be added comparing R counts to FlowJo-reported counts]

### Gate Coordinate Accuracy

Gate boundary coordinates were extracted from each exported v10 XML file and compared to the original gate definitions used to construct the test workspaces (Table 3). For linear rectangle and polygon gates, XML coordinates (in raw data space) were compared directly to the original flowCore gate coordinates. For transformed gates (biexponential, log, arcsinh), the XML data-space coordinates were re-transformed using the same forward transform function and compared to the original transformed-space gate boundaries. Validation was performed using the validate_gate_coordinates.R script (included in inst/scripts/).

Table 3. Gate coordinate accuracy across 8 test scenarios covering 5 gate/transform combinations.

| Test | Population | Gate type | Transform | N coords | Mean deviation | Max deviation | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 02 | FSC_filter | 1D rectangle | Linear | 2 | 0.000000% | 0.000000% | PASS |
| 03 | cells | 2D rectangle | Linear | 4 | 0.000000% | 0.000000% | PASS |
| 04 | singlets | Polygon (6 vertices) | Linear | 6 | 0.000000% | 0.000000% | PASS |
| 06 | FITC_pos | 1D rectangle | Biexponential | 2 | 0.000000% | 0.000000% | PASS |
| 07 | PE_pos | 1D rectangle | Log | 2 | 0.000000% | 0.000000% | known drift^a |
| 08 | APC_pos | 1D rectangle | Arcsinh | 2 | 0.000000% | 0.000000% | PASS |
| 09 | cells | 2D rectangle | Linear | 4 | 0.000000% | 0.000000% | PASS |
| 09 | singlets | 2D rectangle | Linear | 4 | 0.000000% | 0.000000% | PASS |

a Test 07 uses the FlowJo logarithmic transform (decade=6, offset=1), which exhibits coordinate drift on round-trip between platforms. The R-side coordinates are preserved exactly in the exported XML, but the displayed gate position in FlowJo may differ from the original due to differences in how FlowJo and flowCore interpret log transform parameters.

## Standards Compliance

Gating-ML 2.0 compliance was assessed at two levels. First, a structural conformance check verified that gate elements use the correct Gating-ML 2.0 namespace URIs, element names, and required attributes. All 50 applicable checks passed across all 13 test workspaces (Table 4): 13/13 namespace declarations, 10/10 RectangleGate elements, 3/3 PolygonGate elements, 3/3 EllipsoidGate elements, and all transformation elements conformed to the structural requirements of the published specification [4].

Table 4. Gating-ML 2.0 structural conformance results across 13 test workspaces. Each check verifies that required XML elements, namespace URIs, and coordinate attributes conform to the published standard (Spidlen et al. 2012).

| Check category | Applicable | Pass | Fail |
| --- | --- | --- | --- |
| Namespace URIs (gating, transforms, data-type) | 13 | 13 | 0 |
| Linear transform element | 13 | 13 | 0 |
| RectangleGate element | 10 | 10 | 0 |
| PolygonGate element | 3 | 3 | 0 |
| EllipsoidGate element | 3 | 3 | 0 |
| Biex transform element | 3 | 3 | 0 |
| Log transform element | 2 | 2 | 0 |
| Fasinh transform element | 3 | 3 | 0 |
| Overall | 50 | 50 | 0 |

Second, strict XSD validation was attempted using the official Gating-ML 2.0 XSD files [4]. Neither CyFj11-exported WSPs nor an authentic FlowJo v10.10.1 workspace passed strict XSD validation. In both cases, the failure arose from the same cause: FlowJo v10's WSP format uses proprietary transformation element names (transforms:biex, transforms:linear, transforms:log) that are not defined in the Gating-ML 2.0 XSD, which expects transformation content to be wrapped in a standard <transforms:transformation> element. This behavior is an inherent property of the FlowJo v10 format, not a deficiency of CyFj11. CyFj11's output is indistinguishable from authentic FlowJo v10 workspaces with respect to this deviation.

## MIFlowCyt Compliance

The MIFlowCyt standard [6] requires specific metadata for reproducible reporting. We audited CyFj11 against the MIFlowCyt standard to ensure comprehensive reporting of flow cytometry experiment metadata and to facilitate reproducibility. A field-by-field audit is provided in Supplementary File 3.

CyFj11 fully preserves the MIFlowCyt fields within the scope of a gating workspace tool: gate definitions, transformation parameters, compensation matrices, and all FCS keyword data (including $CYT, $DATE, $BTIM, $PnN, $PnS, $TOT, $PAR). FCS keywords are written to the <Keywords> section of every sample in the v10 XML and are available in the GatingSet when the user requests them via the keywords parameter of fj11_to_gatingset(). Fields outside the scope of FlowJo workspaces — experiment purpose, antibody catalog numbers, instrument laser configuration — require manual curation by the user.

Full methodology and intermediate results for both levels of compliance assessment are provided in a dedicated vignette (vignette("gatingml-conformance")).

# Discussion

CyFj11 is designed for researchers who use FlowJo v11 as their primary gating environment and need to perform downstream analysis in R — or who want to export R-derived gating results back into FlowJo for visual review or collaboration. It is particularly useful for high-throughput quality check of manual analysis of high-dimensional flow cytometry files, since it enables quick simultaneous visualization of a given "gated" cell population in numerous samples.

An additional use case is for researchers who cannot use CytoML for importing v10 workspaces due to platform constraints — for example, on macOS with Apple Silicon (ARM), where the CytoML Docker-based C++ components are not maintained. The typical workflow is: gate cells in FlowJo v11, import the workspace into R with fj11_to_gatingset(), apply statistical analysis or batch processing, and optionally return the result to FlowJo via export_flowjo10_workspace().

The package is not intended to replace FlowJo for gating, nor to replace CytoML for users whose workflows are already based on v10 workspaces on standard platforms. It fills the specific gap left by FlowJo's format change: the inability to read v11 workspaces in R and the lack of a maintained, platform-independent v10 export. For v10 import, CyFj11 currently relies on CytoML's FlowJo reader (CytoML::flowjo_to_gatingset()); any limitations of that tool — such as partial support for certain gate types or transformations — also apply to the round-trip comparison workflow.

Several limitations should nevertheless be noted. First, quadrant gates and range gates are not exported to v10 format, because the FlowJo v10 XML schema does not provide direct representations for these gate types; users relying on them will need to reconstruct the gates in FlowJo after export. Second, per-sample gate variations (tailored gates) are not representable in v10 format; export uses the group-level gate. Because fj11_to_gatingset() returns a list of per-sample GatingSet objects rather than a single merged object, users can inspect and work with per-sample gates in R before deciding how to merge them; however, conversion to a single GatingSet via flowWorkspace::merge_list_to_gs() necessarily discards sample-specific variation. Improvements to flowCore to natively represent per-sample gate variations would be a valuable future contribution.

Third, gate positions in log-transformed channels undergo a small but reproducible coordinate drift on round-trip conversion; this is a known issue specific to the FlowJo logarithmic transform parameterization (Test 12 in the validation suite documents this behavior) and affects the displayed gate position, though its impact on population counts is limited to cells near the gate boundary, which are generally less densely populated. Fourth, the validation presented here is based on 13 synthetic test workspaces; real-world workspaces may contain unusual configurations not covered by these tests, though CyFj11 has been applied to large real-world datasets internally without encountering unexpected failures (data not shown).

CytoML [5] provides FlowJo v10 import via XML parsing and export via compiled C++ components in Docker containers. CyFj11 complements this: it adds v11 import (not available in CytoML) and provides a native-R v10 export path that does not require container infrastructure. For users on standard hardware with v10 workspaces, CytoML remains a viable choice. CyFj11's v10 export offers an alternative for users who encounter issues with the CytoML Docker container on current operating systems.

openCyto [3] provides automated gating algorithms that operate on GatingSet objects; CyFj11 is fully compatible with openCyto workflows — a FlowJo v11 workspace can be imported as a starting point for automated gating refinement.

Full v11 export would require either vendor documentation of the FlowJo v11 JSON schema or a substantially larger reverse-engineering effort than was undertaken here. Should BD Biosciences provide official schema documentation, implementing a v11 exporter from GatingSet objects would be a natural extension of CyFj11. In the meantime, the v10 export pathway remains the practical solution for returning R analyses to FlowJo.

Integration with spectral unmixing workflows and mass cytometry (CyTOF) data, as well as support for emerging modalities such as imaging mass cytometry, represent potential future directions as these data types increasingly flow through FlowJo v11.

The package is maintained as open-source software.

# Conclusion

CyFj11 provides a practical, platform-independent solution for FlowJo v11 / R interoperability. It enables researchers to import modern FlowJo v11 workspaces into R with full gate and transformation support, and to return analyzed data to FlowJo via the well-specified v10 XML format. The pure R implementation eliminates dependencies on compiled code and container infrastructure, ensuring compatibility across all platforms supported by R. While the asymmetric design means that v11-specific features such as quadrant gates and per-sample gate variations cannot be fully round-tripped, the core gating information — population hierarchies, gate coordinates, compensation matrices, and transformation parameters — is preserved with high fidelity. CyFj11 is thus beneficial for the many laboratories that rely on high-dimensional flow cytometry and work across both the FlowJo and R environments.

# References

- Patin E, Hasan M, Bergstedt J, Rouilly V, Libri V, Urrutia A, Alanio C, Scepanovic P, Hammer C, Jönsson F, Beitz B, Quach H, Lim YW, Hunkapiller J, Zepeda M, Green C, Piasecka B, Leloup C, Rogge L, Huetz F, Peguillet I, Lantz O, Fontes M, Di Santo JP, Thomas S, Fellay J, Duffy D, Quintana-Murci L, Albert ML; Milieu Intérieur Consortium. Natural variation in the parameters of innate immune cells is preferentially driven by genetic factors. Nat Immunol. 2018 Mar;19(3):302-314. doi: 10.1038/s41590-018-0049-7. Epub 2018 Feb 23. Erratum in: Nat Immunol. 2018 Jun;19(6):645. doi: 10.1038/s41590-018-0105-3. PMID: 29476184.

- FlowJo™ Software Version 10 & 11. Becton, Dickinson and Company; 2023.

- Ellis B, Haaland P, Hahne F, Le Meur N, Gopalakrishnan N, Spidlen J, Jiang M, Finak G (2026). _flowCore: flowCore: Basic structures for flow cytometry data. doi:10.18129/B9.bioc.flowCore <https://doi.org/10.18129/B9.bioc.flowCore>, R package version 2.22.1, <https://bioconductor.org/packages/flowCore>.

- Finak G, Frelinger J, Jiang W, Newell EW, Ramey J, Davis MM, et al. (2014) OpenCyto: An Open Source Infrastructure for Scalable, Robust, Reproducible, and Automated, End-to-End Flow Cytometry Data Analysis. PLoS Comput Biol 10(8): e1003806. https://doi.org/10.1371/journal.pcbi.1003806.

- Spidlen, J., Moore, W., & Brinkman, R. R. (2015). ISAC's Gating-ML 2.0 data exchange standard for gating description. Cytometry Part A, 87(7), 683-687. https://doi.org/10.1002/cyto.a.22690.

- Finak G, Jiang W, Gottardo R. CytoML for cross-platform cytometry data sharing. Cytometry A. 2018 Dec;93(12):1189-1196. doi: 10.1002/cyto.a.23663. PMID: 30551257; PMCID: PMC6443375.

- Lee JA, Spidlen J, Boyce K, Cai J, Crosbie N, Dalphin M, Furlong J, Gasparetto M, Goldberg M, Goralczyk EM, Hyun B, Jansen K, Kollmann T, Kong M, Leif R, McWeeney S, Moloshok TD, Moore W, Nolan G, Nolan J, Nikolich-Zugich J, Parrish D, Purcell B, Qian Y, Selvaraj B, Smith C, Tchuvatkina O, Wertheimer A, Wilkinson P, Wilson C, Wood J, Zigon R; International Society for Advancement of Cytometry Data Standards Task Force; Scheuermann RH, Brinkman RR. MIFlowCyt: the minimum information about a Flow Cytometry Experiment. Cytometry A. 2008 Oct;73(10):926-30. doi: 10.1002/cyto.a.20623. PMID: 18752282; PMCID: PMC2773297.

# Supplementary Files

Supplementary File 1: FlowJo v10 XML vs v11 JSON Architecture Comparison

Supplementary File 2: Full Validation Methodology and Test Scenario Descriptions

Supplementary File 3: MIFlowCyt Compliance Audit

Supplementary Table S1: Complete Population Count Results for All 13 Validation Scenarios

Supplementary File 4: Gating-ML 2.0 Conformance Validation Script Output

# Annex A: Technical Implementation Details

This annex contains information for readers interested in the technical architecture of CyFj11. It is not required for understanding the package’s capabilities or workflow.

## A.1 FlowJo workspace format comparison

FlowJo v10 (.wsp) is a single XML file following the ISAC Gating-ML 2.0 standard. The hierarchical structure contains <Workspace>, <SampleList>, <Groups>, and <Population> elements. Gates are expressed as Gating-ML elements (<gating:RectangleGate>, <gating:PolygonGate>, etc.) with coordinate transformations specified in dedicated elements. The format is human-readable, well-documented, and validated by the community.

FlowJo v11 (.flowjo) is a ZIP archive containing JSON files (analysis-*.json, manifest.txt). The JSON schema uses UUID-based cross-referencing: populationDefinitions (gate geometry), populations (sample-specific instances with statistics), dataSources (FCS file references), and groups (sample collections) are separate entities linked by UUIDs. Transformations are embedded within axis definitions rather than stored separately. The format is proprietary and not publicly documented.

Key structural differences that constrain the design of CyFj11:

| Aspect | v10 (XML) | v11 (JSON) |
| --- | --- | --- |
| Format | Single XML file | ZIP with multiple JSON files |
| Standard | Gating-ML 2.0 (published) | Proprietary (undocumented) |
| Gate representation | Nested XML elements | UUID-linked JSON objects |
| Transformation storage | Separate elements | Embedded in axis definitions |
| Per-sample variation | Not supported | desyncTable mechanism |
| Population hierarchy | Nested XML | Parent/child UUID references |

The v11 JSON schema contains  top-level entity types including: groups, dataSources, populationDefinitions, populations, compoundParameterSets, compoundPopulations, paramsetDefinitions, platforms, cytometers, and analysisRoot. Each entity maintains cross-reference sections (parents, children, results) that link to other entities by UUID. A single group entity, for example, carries parent and child references to all other entity types, requiring that any write operation maintain global UUID consistency across the entire workspace graph. By contrast, the v10 XML schema uses nested containment: a <Population> element that contains its <Gate> is self-contained and does not require references to be resolved elsewhere in the document.

The complexity of constructing a valid v11 export — requiring the generation of a consistent UUID dependency graph across all 10 entity types — is the primary technical barrier to implementing v11 export.

## A.2 Gate type conversion

### Rectangle gates

v11 rectangle gates define min/max boundaries per parameter. On import, these are converted to flowCore::rectangleGate objects. On export to v10, they are written as <gating:RectangleGate> elements with <gating:dimension> children specifying gating:min and gating:max attributes.

1D rectangle gates (range gates in v11 terminology) are imported as rectangleGate objects with one bounded and one unbounded dimension. They are exported using the same <gating:RectangleGate> structure.

### Polygon gates

v11 polygon gates store vertex coordinates as arrays of [x, y] pairs. On import, vertices are converted to a flowCore::polygonGate matrix. Coordinate inverse-transformation is applied to map from display space to data space. On export, vertices are written as <gating:vertex> elements within <gating:PolygonGate>.

### Ellipsoid gates

v11 ellipsoid gates are parameterized by a mean vector and covariance matrix. On import, these are converted to flowCore::ellipsoidGate objects. On export to v10, eigenvalue decomposition is applied to derive rotation angle and semi-axis lengths for the <gating:EllipsoidGate> representation. This decomposition introduces minor numerical differences relative to the original v11 gate.

### Boolean gates

v11 Boolean gates reference other gates by UUID and specify AND/OR/NOT combinations. On import, UUID references are resolved to population paths and a flowCore::booleanFilter is constructed. On export, Boolean logic is preserved in <gating:BooleanGate> elements referencing gates by population path.

### Quadrant gates

v11 quadrant gates define dividing lines on two axes, producing four quadrant populations. On import, CyFj11 converts each quadrant into a flowCore::quadGate object. On export to v10, quadrant gates have no direct XML representation in the format CyFj11 generates and are excluded with a warning.

## A.3 Transformation parameter mapping

All transformations are extracted from v11 axis definitions and converted to the corresponding flowCore transformation objects on import. On export to v10, they are written as <transforms:*> elements with the appropriate parameter attributes.

Display-space versus data-space values: An important distinction when working with FlowJo transformations is that FlowJo displays gate coordinates in display space (the transformed, scaled axis values shown on screen), while flowCore and the Gating-ML 2.0 standard store gate coordinates in data space (raw channel values). CyFj11 handles the inverse transformation from display space to data space during import for gate coordinate extraction, and applies the forward transformation during export. Users inspecting gate coordinates directly in the v10 XML should be aware that these are in data space and will differ from the values shown on FlowJo’s axis.

| Transformation | v11 parameter names | v10 / flowCore parameter names | Notes |
| --- | --- | --- | --- |
| Biexponential | T, W, M, A | T, W, M, A | Direct mapping |
| Logicle | T, W, M, A | T, W, M, A | Direct mapping |
| Log | decades, offset | decades, offset | Base 10 assumed |
| Arcsinh | T, A | length, maxRange, T, A, M, W | Parameter extension required |
| Linear | min, max | minRange, maxRange | Direct mapping |

## A.4 Package architecture

CyFj11 is organized into the following R source modules:

| File | Responsibility |
| --- | --- |
| archive.R | ZIP extraction, JSON parsing, manifest reading |
| conversion.R | Per-sample GatingSet list construction from parsed workspace |
| export-flowjo10.R | Gating-ML 2.0 XML generation |
| gates.R | Gate type detection, coordinate extraction, conversion |
| transformations.R | Transformation extraction and format conversion |
| compensation.R | Compensation matrix extraction and validation (see also 8 Color PBMC real-world test) |
| populations.R | Population hierarchy traversal and gate tree construction |
| file-search.R | FCS file resolution from FlowJo URI references |
| helpers.R | Utility functions, UUID validation |

The exported functions are: read_flowjo11_workspace(), fj11_to_gatingset(), export_flowjo10_workspace(), and pretty_print_flowjo() (extracts and formats a .flowjo workspace to a human-readable JSON text file, useful for debugging). Three utility functions are also exported: set_verbose(TRUE/FALSE) enables or disables verbose progress logging; get_verbose() returns the current verbosity state; and %||% is a null-coalescing operator that returns its left-hand argument if non-NULL, otherwise the right-hand argument (analogous to ?? in other languages).

## A.5 Validation methodology

### Test workspace construction

Thirteen test workspaces were constructed programmatically using the roundtrip_experiment.Rmd script included with the package. Synthetic FCS files were generated using log-normal bimodal distributions to simulate realistic fluorescence measurements: scatter channels (FSC-A, FSC-H, SSC-A, SSC-H) were drawn from normal distributions (FSC-A: μ = 120,000, σ = 30,000; SSC-A: μ = 80,000, σ = 20,000), and fluorescence channels were drawn from a mixture of two log-normal components (negative population: ~70–80% of events; positive population: ~20–30% of events) to produce realistic positive/negative separations. All random seeds were fixed for reproducibility.

Gate coordinates were specified to enclose known fractions of the simulated distributions. Transformation parameters were set to standard FlowJo defaults: biexponential with channelRange = 4096, maxValue = 262,144, pos = 4.5, neg = 0; logicle with T = 262,144, W = 0.5, M = 4.5, A = 0; arcsinh with T = 262,144, M = 4.5, A = 0.

### Metrics

- Population count preservation: absolute count difference and percentage difference between R-side GatingSet count and FlowJo-reported count after importing the exported v10 workspace.

- Gate coordinate accuracy: mean and maximum deviation of gate boundary coordinates as a percentage of the axis range, computed by comparing coordinates in the exported v10 XML to the original gate definition.

- Transformation fidelity: Kolmogorov–Smirnov D-statistic comparing the distribution of transformed values computed by flowCore and by FlowJo for identical parameter values and FCS files.

- Boolean gate accuracy: percentage agreement in per-cell membership for converted Boolean gates (AND, OR, NOT), computed by comparing R-side and FlowJo-side population counts.

### Acceptance criteria

| Metric | Acceptance criterion |
| --- | --- |
| Population count difference | ≤ 0.5% absolute |
| Gate coordinate deviation | ≤ 0.1% of axis range (95th percentile) |
| KS D-statistic (transformations) | < 0.05 for ≥ 95% of comparisons |
| Boolean membership agreement | 100% |

# Annex B: Test Scenario Descriptions

All test scenarios use synthetic FCS data generated by the validation suite in roundtrip_experiment.Rmd. Each test directory contains the FCS file(s), an exported v10 workspace (export.wsp), and a description file (test_info.txt).

| Test | Samples | Gate hierarchy | Gate types | Transforms | Known issues |
| --- | --- | --- | --- | --- | --- |
| 01 | 1 | Root only | None | Linear | None |
| 02 | 1 | 1 level | 1D rectangle (FSC-A, 60k–180k) | Linear | None |
| 03 | 1 | 1 level | 2D rectangle (FSC-A, SSC-A) | Linear | None |
| 04 | 1 | 1 level | Polygon on FSC-A vs FSC-H (6 vertices) | Linear | None |
| 05 | 1 | 1 level | Ellipsoid on FSC-A vs SSC-A | Linear | Minor coordinate difference expected from eigenvalue decomposition |
| 06 | 1 | 1 level | 1D rectangle on FITC-A | Biexponential (channelRange=4096, pos=4.5, neg=0) | None |
| 07 | 1 | 1 level | 1D rectangle on PE-A | Log (t=262144, m=4.5) | Gate position drifts on round-trip; minor count difference expected |
| 08 | 1 | 1 level | 1D rectangle on APC-A | Arcsinh (T=262144, M=4.5, A=0) | None |
| 09 | 1 | 2 levels: cells → singlets | 2D rectangle × 2 | Linear | None |
| 10 | 1 | 2 levels: root → {FSC_gate, SSC_gate, both} | 2× 1D rectangle + Boolean AND | Linear | None |
| 11 | 22 | 2 levels: cells → {FITC_pos, APC_pos, singlets, double_pos, either_pos} | 2D rect + 2× 1D rect + polygon + 2× boolean | Biexponential (FITC-A) + arcsinh (APC-A) + linear | Real patient data |
| 12 | 1 | 2 levels: root → {cells, scatter_ellipse}; cells → {FITC_hi, PE_hi, double_hi, either_hi, neither} | 2D rect + ellipsoid + 2× range + 3× boolean (AND/OR/NOT) | Log (PE-A) + biexponential (FITC-A) + linear | Log-transform gate drift; NOT boolean gate tested |
| 13 | 1 | 3 levels: live → {singlets, FITC_PE_gate}; singlets → {FITC_pos, APC_pos, double_pos, not_double} | Polygon + 2D rect + ellipsoid + 3× 1D rect + 2× boolean | Arcsinh (FITC-A, PE-A, APC-A) + linear | 3-level depth; NOT boolean gate tested |

# Annex C: MIFlowCyt Compliance Checklist

The MIFlowCyt standard [6] specifies minimum information required for reproducible reporting of flow cytometry experiments. The table below audits each MIFlowCyt field category against the CyFj11 pipeline: (1) whether the information is present in a FlowJo v11 workspace, (2) whether fj11_to_gatingset() imports it into the R GatingSet, and (3) whether export_flowjo10_workspace() writes it to the v10 XML.

Symbols: ✓ = fully preserved; ~ = partial / conditional; ✗ = not preserved; N/A = not applicable to software tools.

## C.1 Experiment Overview

| MIFlowCyt item | In v11 workspace? | Imported to R? | Exported to v10? | Notes |
| --- | --- | --- | --- | --- |
| Purpose / hypothesis | ✗ | ✗ | ✗ | Not stored in FlowJo workspaces or FCS files |
| Keywords / domain terms | ✗ | ✗ | ✗ | Not stored in workspace |
| Number of experiments | N/A | N/A | N/A | Single-workspace tool |

## C.2 Primary Specimen and Flow Sample

These fields are stored in FCS keyword fields when present; availability depends on the instrument and acquisition software used to generate the FCS files.

| MIFlowCyt item | FCS keyword | In v11 workspace? | Imported to R? | Exported to v10? | Notes |
| --- | --- | --- | --- | --- | --- |
| Organism / cell type | $CELLS, $SRC | ~ | ~ | ~ | Available if present in FCS keywords; extracted via keywords parameter |
| Sample source / tissue | $SRC | ~ | ~ | ~ | As above |
| Sample preparation description | — | ✗ | ✗ | ✗ | Not stored in FCS standard |
| Collection date and time | $DATE, $BTIM, $ETIM | ✓ | ~ | ✓ | In FCS keywords; written to v10 <Keywords> section; |
| Operator | $OP | ✓ | ~ | ✓ | As above |
| Institution | $INST | ~ | ~ | ~ | Present if recorded by acquisition software |
| Sample number / well ID | $SMNO, $WELLID | ~ | ~ | ~ | Present if recorded |

## C.3 Reagents and Staining

| MIFlowCyt item | FCS keyword | In v11 workspace? | Imported to R? | Exported to v10? | Notes |
| --- | --- | --- | --- | --- | --- |
| Fluorochrome names | $PnS | ✓ | ✓ | ✓ | Channel stain descriptions; in parameter keywords |
| Channel names (detector) | $PnN | ✓ | ✓ | ✓ | Required FCS keyword; always present |
| Antibody clone / catalog | — | ✗ | ✗ | ✗ | Not a standard FCS field |
| Antibody concentration | — | ✗ | ✗ | ✗ | Not a standard FCS field |
| Viability dye | — | ✗ | ✗ | ✗ | Convention only; not standardized |

## C.4 Instrument

| MIFlowCyt item | FCS keyword | In v11 workspace? | Imported to R? | Exported to v10? | Notes |
| --- | --- | --- | --- | --- | --- |
| Cytometer manufacturer and model | $CYT | ✓ | ~ | ✓ | In FCS keywords; written to v10 XML <Keywords>; |
| Excitation laser wavelengths | — | ✗ | ✗ | ✗ | Not in FCS standard; instrument-specific |
| Detector/filter configuration | $PnF (non-standard) | ~ | ~ | ~ | Instrument-specific; not always recorded |
| Amplifier gain / voltage | $PnV, $PnG | ~ | ~ | ~ | Instrument-specific; present if recorded |

## C.5 Acquisition

| MIFlowCyt item | FCS keyword | In v11 workspace? | Imported to R? | Exported to v10? | Notes |
| --- | --- | --- | --- | --- | --- |
| Trigger / threshold channel | — | ✗ | ✗ | ✗ | Not standardized in FCS |
| Threshold value | — | ✗ | ✗ | ✗ | Not standardized in FCS |
| Collection volume / events | $TOT | ✓ | ✓ | ✓ | Total event count; used in sample naming |
| Number of parameters | $PAR | ✓ | ✓ | ✓ | Required FCS keyword |
| FCS file format version | FCSversion | ✓ | ✓ | ✓ | Written as keyword |
| Data type | $DATATYPE | ✓ | ~ | ✓ | In FCS keywords |
| Byte order | $BYTEORD | ✓ | ~ | ✓ | In FCS keywords |

## C.6 Data Analysis

| MIFlowCyt item | Source | In v11 workspace? | Imported to R? | Exported to v10? | Notes |
| --- | --- | --- | --- | --- | --- |
| Analysis software name | Workspace JSON | ✓ | ✗ | ✗ | FlowJo version in schemaVersion; not propagated to GatingSet |
| Analysis software version | Workspace JSON | ✓ | ✗ | ✗ | As above |
| Gate names | populationDefinitions | ✓ | ✓ | ✓ | Fully extracted into GatingHierarchy |
| Gate coordinates | gateDefinition | ✓ | ✓ | ✓ | All gate types including per-sample variations |
| Gate hierarchy | parent/child UUIDs | ✓ | ✓ | ✓ | Full population tree preserved |
| Transformation type | axis parameterSpec | ✓ | ✓ | ✓ | biexp, logicle, log, arcsinh, linear |
| Transformation parameters | axis parameterSpec | ✓ | ✓ | ✓ | T, W, M, A parameters for each channel |
| Compensation matrix | platforms.spilloverMatrix | ✓ | ✓ | ✓ | Extracted from multiple possible sources |
| Compensation controls | Compensation group | ✓ | ~ | ~ | Group identified; FCS files resolved |
| Population statistics | populations results | ✓ | ✓ | ✓ | Counts, frequencies after gate execution |
| Per-sample gate variations | desyncTable | ✓ | ✓ | ~ | Imported; merged to group gate on export |

## C.7 Summary

| Category | Import fidelity | Export fidelity | Key limitation |
| --- | --- | --- | --- |
| Experiment overview | ✗ | ✗ | Not stored in FlowJo workspaces |
| Sample metadata | ~ | ~ | FCS keyword fields are preserved but not surfaced to pData by default; use keywords parameter |
| Reagents / staining | ~ | ~ | $PnS (stain names) preserved; antibody details not in FCS standard |
| Instrument | ~ | ~ | $CYT and parameter keywords preserved in XML; not in pData by default |
| Acquisition | ✓ | ✓ | Core fields ($TOT, $PAR, $PnN) always preserved |
| Data analysis (gates) | ✓ | ✓ | Complete: hierarchy, coordinates, types, transforms |
| Compensation | ✓ | ✓ | Full matrix from multiple extraction paths |
| Software provenance | ~ | ✗ | FlowJo version available in v11 JSON; not written to v10 XML |

Overall assessment: CyFj11 fully preserves the MIFlowCyt fields that are within the scope of a gating workspace tool — gate definitions, transformations, compensation matrices, and FCS keyword data.